# ESP32-S3 Dual ADS1115 Sensor System

## Functional Specification Document (FSD)

## 1. Overview

This document defines the functional, electrical, and wiring specifications for an ESP32-S3–based sensing system used in an RV environment. The system integrates:

- Two ADS1115 16-bit ADC modules

- ZMPT101B AC voltage sensors

- SCT-013 current sensors

- I²C communication bus

- Stable power-up circuitry including pull-ups, decoupling capacitors, and CHIP_PU RC delay

The goal is to ensure reliable measurement, clean power delivery, and stable boot behavior.

## 2. System Architecture

### 2.1 High-Level Block Diagram

- ESP32-S3 (main controller)

  - Provides 3.3 V logic and I²C master

  - Reads voltage and current data from ADS1115 modules

- ADS1115 #1 (Voltage sensing)

  - Connected to ZMPT101B modules

  - I²C address: 0x48

- ADS1115 #2 (Current sensing)

  - Connected to SCT-013 sensors

  - I²C address: 0x49

- Power subsystem

  - 5 V input → ESP32 VIN

  - ESP32 3V3 output → ADS1115 VDD

  - Decoupling capacitors on VIN and ADS1115 modules

  - CHIP_PU RC delay for stable boot

## 3. Electrical Specifications

### 3.1 Power Input

| Source | Voltage | Notes |
| --- | --- | --- |
| USB-C / Micro-USB | 5 V | Easiest method; powers onboard regulator |
| VIN pin | 5 V regulated | Add bulk capacitors at VIN |
| 3V3 pin | 3.3 V regulated | Must be clean and stable; bypasses onboard LDO |

### 3.2 Power Conditioning Requirements

- VIN bulk capacitors:

  - 47 µF electrolytic

  - 100 nF ceramic

- ADS1115 VDD decoupling:

  - 10 µF electrolytic

  - 100 nF ceramic

- Placement: As close as possible to each module’s power pins.

### 3.3 ESP32 CHIP_PU (EN) Circuit

| Component | Value | Purpose |
| --- | --- | --- |
| Pull-up resistor | 10 kΩ → 3.3 V | Keeps ESP32 enabled |
| Capacitor | 100 nF (up to 1 µF optional) | RC delay for stable boot |
| Reset button | Optional | Pulls CHIP_PU to GND |

RC Delay Formula: [ V(t) = V_{CC} \times (1 - e^{-t/RC}) ]

- R = 10 kΩ

- C = 100 nF → ~1 ms delay

- C = 1 µF → ~10 ms delay

## 4. I²C Bus Specifications

### 4.1 Bus Voltage

- All I²C devices operate at 3.3 V logic.

- No level shifters required.

### 4.2 Pull-Up Resistors

| Line | Resistor | Notes |
| --- | --- | --- |
| SDA | 4.7 kΩ → 3.3 V | Recommended |
| SCL | 4.7 kΩ → 3.3 V | Recommended |

Range: 3.3 kΩ (strong) to 10 kΩ (weak)

Only one set of pull-ups is required for the entire bus.

### 4.3 I²C Addresses

| Module | Address Pin | Address |
| --- | --- | --- |
| ADS1115 #1 | ADDR → GND | 0x48 |
| ADS1115 #2 | ADDR → VDD | 0x49 |

### 4.4 Recommended Bus Speed

- Default: 100 kHz

- Optional: 400 kHz Fast Mode (if wiring is short and clean)

## 5. Sensor Wiring Specifications

### 5.1 ZMPT101B Voltage Sensors → ADS1115 #1

| Sensor | ADS1115 Channel |
| --- | --- |
| ZMPT #1 | A0 |
| ZMPT #2 | A1 |
| ZMPT #3 | A2 |

### 5.2 SCT-013 Current Sensors → ADS1115 #2

| Sensor | ADS1115 Channel |
| --- | --- |
| SCT-013 #1 | A0 |
| SCT-013 #2 | A1 |
| SCT-013 #3 | A2 |

Notes:

- Some SCT-013 models require an external burden resistor.

- ZMPT101B modules require calibration for accurate AC measurement.

## 6. ESP32-S3 Wiring Summary

### 6.1 I²C Pins

| ESP32-S3 Pin | Function |
| --- | --- |
| GPIO 8 | SDA |
| GPIO 9 | SCL |

### 6.2 Power Pins

| ESP32 Pin | Destination |
| --- | --- |
| 3V3 | ADS1115 VDD |
| GND | All modules |
| VIN | 5 V input |

## 7. Firmware Requirements

### 7.1 ADS1115 Configuration

- Gain: GAIN_ONE (±4.096 V range)

- Mode: Single-ended reads

- Sampling: 860 SPS recommended for AC waveform sampling

### 7.2 Example Firmware Structure

- Initialize I²C

- Initialize both ADS1115 modules

- Read voltage channels

- Read current channels

- Apply calibration factors

- Output via serial or Wi-Fi

## 8. Reliability & Stability Requirements

### 8.1 Power Stability

- Bulk capacitors required at VIN

- Decoupling capacitors required at each ADS1115

### 8.2 Boot Stability

- CHIP_PU pull-up + RC delay mandatory

- Keep capacitor ground trace short

### 8.3 Noise Reduction

- Keep sensor wiring short

- Avoid routing I²C near AC lines

- Use twisted pair for SCT-013 if possible

## 9. Mechanical & Installation Notes

- Secure ZMPT101B modules to avoid AC interference

- Clamp SCT-013 around only one conductor (hot or neutral)

- Keep ADS1115 modules close to ESP32 to minimize I²C capacitance

## 10. Revision History

| Version | Date | Notes |
| --- | --- | --- |
| 1.0 | Feb 2026 | Initial FSD generated from wiring and power design documents |

## 11. Appendix

### 11.1 Recommended Wiring Diagram (Text Form)

5V → VIN (ESP32)

└─ 47µF + 100nF → GND

ESP32 3V3 → ADS1115 #1 VDD

ESP32 3V3 → ADS1115 #2 VDD

GPIO 8 (SDA) → Both ADS1115 SDA

GPIO 9 (SCL) → Both ADS1115 SCL

3.3V → 4.7kΩ → SDA

3.3V → 4.7kΩ → SCL

ADS1115 #1 ADDR → GND (0x48)

ADS1115 #2 ADDR → VDD (0x49)

CHIP_PU → 10kΩ → 3.3V

CHIP_PU → 100nF → GND

This FSD consolidates all wiring, electrical, and operational requirements into a single authoritative reference for your ESP32-S3 RV sensor system.